# PROTOVOLT Panel Box Testing Kit
Technical Documentation

Version: 1.0
Purpose: Reference document for building, testing and maintaining the PROTOVOLT Panel Box Testing Kit.

# 1. System Overview

The testing kit is built around an ESP32-S3 Waveshare 2.8-inch Touch LCD. It provides PLC communication over RS485 (Modbus RTU), power monitoring using an INA260, NFC identification using a PN532 reader, relay control, and automated panel box testing.

# 2. Hardware Requirements

| Component | Model | Purpose |
| --- | --- | --- |
| Controller | ESP32-S3 Waveshare Touch LCD 2.8" | Main controller and HMI |
| RS485 | 7Semi Breakout | PLC communication |
| Power Monitor | INA260 (I2C) | Voltage monitoring |
| NFC Reader | PN532 (I2C) | Read NFC tags/stickers |
| Power Supply | 7Semi DC-DC 5V | 5V regulation |

# 3. Software Requirements

| Library | Version |
| --- | --- |
| ESP32 by Espressif Systems | 3.0.7 |
| LVGL | 8.3.10 |
| Adafruit_INA260 | 1.5.3 |
| Adafruit_PN532 | 1.3.4 |
| ModbusMaster | 2.0.1 |

# 4. Pin Configuration

| Interface | Signal | GPIO |
| --- | --- | --- |
| I2C | SDA | 11 |
| I2C | SCL | 10 |
| RS485 | RXD2 | 44 |
| RS485 | TXD2 | 43 |
| RS485 | EN485 | 18 |

# 5. PLC Configuration

| Parameter | Value |
| --- | --- |
| Protocol | Modbus RTU |
| Slave ID | 1 (per source code) |
| Baud Rate | 115200 |
| Parity | None |
| Stop Bits | 1 |

# 6. Voltage Test

| Minimum | Maximum |
| --- | --- |
| 11.90 V | 12.90 V |

PASS when measured voltage is within the above range.

# 7. Screen Flow

1. NFC Screen - Scan NFC tag and display UID.

2. PLC Check Screen - Verify PLC communication or scan baud rate.

3. Test Screen - Execute Power, Trigger, TB15 and Interlock tests.

4. Relay Control Screen - Toggle relays R0-R7.

# 8. Automated Tests

| Test | Description | Pass Condition |
| --- | --- | --- |
| Power Test | INA260 voltage | 11.90-12.90 V |
| Trigger Check | Digital Input 0 | DI0 Active |
| TB15 Check | Digital Input 1 | DI1 Active |
| Interlock Check | Digital Input 2 | DI2 Active |

# 9. Relay Mapping

| Relay | Coil Address |
| --- | --- |
| R0 | 0 |
| R1 | 1 |
| R2 | 2 |
| R3 | 3 |
| R4 | 4 |
| R5 | 5 |
| R6 | 6 |
| R7 | 7 |

# 10. Wiring Summary

I2C Bus:
GPIO11 -> SDA
GPIO10 -> SCL
Connected devices: INA260 and PN532.

RS485:
GPIO43 -> TXD2
GPIO44 -> RXD2
GPIO18 -> EN485.

# 11. Troubleshooting

| Issue | Possible Cause | Action |
| --- | --- | --- |
| PLC not detected | RS485 wiring / baud mismatch | Check RX/TX, EN485 and PLC settings |
| No voltage | INA260 not detected | Verify I2C wiring |
| NFC not working | PN532 connection | Check SDA/SCL and module power |
| Relay failure | Communication issue | Verify Modbus coil mapping |

# 12. Future Improvements

Add wiring diagrams.

Include Modbus register map.

Add SD card logging.

Support OTA firmware updates.

Generate automatic test reports.

# Reference

This report was prepared from the project configuration and source material provided in the conversation.